AI Visual strike module

Technical Specification Sheet

Overview

- The AI Visual Strike Module is an AI visual perception tracking kit designed specifically for fighter UAV, integrating high-performance target recognition and dynamic tracking. By integrating deep learning algorithms with high-performance intelligent chips, it supports multi-type target detection, with a range of up to 300 meters for human-vehicle target recognition.

Package list

- AI main board *1

- Single camera payload *1

- Power cable *1

- Serial comm.port*1

- Network comm.port*1

Serial comm.cable

Network comm. cable

Power cable

Camera

AI main board

AI visual strike module packing list

Product Dimensions

AI image processing board

Dimensions (length width height) : 85x56x15mm

Single camera payloa

Size: 41.8 x 57 x 22.5mm

Product Interface Definition

Fan outlet

Backup battery port

I/O port

Power input port

Serial Port 2

Network port

Serial Port 1

CAM2

Debug Port

CAM1

Serial 3

TF Card slot

Serial 6

Serial 4+5

USB_OTG

USB_HOST

AI image processing board interface

AI Image processing board pin definitions

- Network port

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - RX+ | - Default |
| - P2 | - RX- | - Default |
| - P3 | - TX+ | - Default |
| - P4 | - TX- | - Default |

- Serial Port 1

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - VCC | - 5V+ |
| - P2 | - UART3_RX | - 3.3 V |
| - P3 | - UART3_TX | - 3.3 V |
| - P4 | - GND | - GND |

- Serial Port 2

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - VCC | - 5V+ |
| - P2 | - UART2_RX | - 3.3 V |
| - P3 | - UART2_TX | - 3.3 V |
| - P4 | - GND | - GND |

- Power input port

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - VCC | - 12V+ |
| - P2 | - GND | - GND |

- USB_HOST

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - GND | - GND |
| - P2 | - GND | - GND |
| - P3 | - USB2.0 _DP | - Default |
| - P4 | - USB2.0 _DM | - Default |
| - P5 | - VBUS | - 5V+ |

- USB_OTG

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - VBUS | - GND |
| - P2 | - USB2.0 _DM | - Default |
| - P3 | - USB2.0 _DP | - Default |
| - P4 | - GND | - GND |

- Serial Port 3

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - VCC | - 5V+ |
| - P2 | - UART6_RX | - 3.3 V |
| - P3 | - UART6_TX | - 3.3 V |
| - P4 | - GND | - GND |

- Serial port 4+5

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - UART11_RX | - 3.3 V |
| - P2 | - UART11_TX | - 3.3 V |
| - P3 | - UART10_RX | - 3.3 V |
| - P4 | - UART10_TX | - 3.3 V |
| - P5 | - GND | - GND |
| - P6 | - VCC | - 12V+ |

- Serial Port 6

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - UART9_RX | - 12V+ |
| - P2 | - UART9_TX | - 3.3 V |
| - P3 | - GND | - GND |

- CAM1/CAM2

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - D2N | - Default |
| - P2 | - D3N | - Default |
| - P3 | - D2P | - Default |
| - P4 | - D3P | - Default |
| - P5 | - D0N | - Default |
| - P6 | - GND | - GND |
| - P7 | - D0P | - Default |
| - P8 | - GND | - GND |
| - P9 | - GND | - GND |
| - P10 | - D1N | - Default |
| - P11 | - CLKN | - Default |
| - P12 | - D1P | - Default |
| - P13 | - CLKP | - Default |
| - P14 | - MCLK_24M | - 1.8 V |
| - P15 | - GND | - GND |
| - P16 | - GND | - GND |
| - P17 | - PDN | - 1.8 V |
| - P18 | - I2C_SDA | - 1.8 V |
| - P19 | - RST | - 1.8 V |
| - P20 | - I2C_SCL | - 1.8 V |
| - P21 | - VCCIO | - 1.8 V + |
| - P22 | - GND | - GND |
| - P23 | - VCCAF | - 2.8 V + |
| - P24 | - VCC | - 1.2 V + |
| - P25 | - VCC_AF | - 2.8 V + |
| - P26 | - VCC | - 1.2 V + |

- Backup battery port

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - GND | - GND |
| - P2 | - VCC_RTC | - 3.0 V + |

- Fan outlet

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - GND | - GND |
| - P2 | - VCC | - 5.0 V + |

- I/O port

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - VCC | - 5.0 V + |
| - P2 | - I/O1 | - 3.3 V |
| - P3 | - I/O2 | - 3.3 V |
| - P4 | - I/O3 | - 3.3 V |
| - P5 | - I/O4 | - 3.3 V |
| - P6 | - I/O5 | - 3.3 V |
| - P7 | - GND | - GND |

- TF card slot

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - Default | - Default |
| - P2 | - Default | - Default |
| - P3 | - Default | - Default |
| - P4 | - Default | - Default |
| - P5 | - Default | - Default |
| - P6 | - Default | - Default |
| - P7 | - Default | - Default |
| - P8 | - Default | - Default |
| - P9 | - Default | - Default |

Features

- Long-distance identification

- Equipped with high-precision optical modules, the maximum recognition distance for human-vehicle targets can reach up to 300 meters.

- Multi-type target recognition

- It can autonomously recognize common targets such as people and vehicles, and supports box selection locking and tracking of hundreds of target types, including: buildings, Windows, doors, etc. The recognition capability can be expanded through continuous dataset training to ensure the accuracy of task execution.

- Target box selection

- Supports framing targets in visible light images for target tracking and striking.

- Auto-strike

- After determining the target, the target strike can be automatically completed through visual guidance without the need for manual intervention.

- Anti-interference strikes

- During the strike, even if the drone's communication link or GPS is disrupted, the target can be precisely struck through visual guidance.

- Lightweight integration

- Full modular design significantly reduces the cost of deploying AI capabilities.

Technical Parameters

| Performance parameters |  |
| --- | --- |
| Support detection of target recognition types | Default human-vehicle, support lock tracking of multiple types of targets |
| Support target recognition distance | Vehicles: 300m People: 300m |
| Maximum strike speed | 90km/h |
| Overall parameters of the AI image processing board |  |
| Master computing power (TOPS) | 6 |
| Control Protocol | Private protocol |
| Support flight control | PX4 |
| Input voltage (V) | 12 |
| Size (length, width, height mm) | 85*56*15 |
| Mounting hole positions (mm) | 3.5 * 4 |
| Gimbal parameters |  |
| Gimbal type | Single-axis mechanical stabilization |
| Image stabilization accuracy | ± 0.005 ° |
| Maximum rotation range | Pitch: ±120° |
| Maximum rotational speed | ±1500°/s |
| Dimensions (including shock-absorbing components | 41.8 x 57 x 22.5mm |
| Installation method | Hoisting |
| Camera parameters |  |
| Sensor (Inch) | 1/2.9 |
| Focal length (mm) | 5mm |
| FOV | (D)30° |
| Video output format | 1080P/60fps |
| Camera size (length, width, height mm) | 18.99 * 18.99 * 26.77 |

Notes

Daily maintenance

- Dust-proof and stain-proof

- Avoid contamination of the lens with chemicals, oil stains, etc. during use. Wipe and clean the lens surface with a lens cloth immediately after use.

- Do not wipe the optical lens directly with your hands or rough cloth. It is recommended to use a professional lens cleaning kit or a lint-free cloth and alcohol for cleaning.

- Moisture-proof and water-resistant

- Avoid prolonged use in rain, snow, high humidity conditions. If in a damp environment, wipe the surface with a dry soft cloth after work and store in a dry environment.

- Do not soak or rinse the equipment directly.

- Anti-collision

- Do not use sharp or hard objects to strike the lens, as this can cause the lens to break and affect its use.

Instructions for Use

- Do not disassemble by yourself

- Non-professionals are not allowed to disassemble core components of the pod (such as optical modules, circuit boards), which may result in damage or disqualification from warranty.

- Anomaly handling

- If the device shows problems such as overheating, noise, image distortion, etc., stop using it immediately and contact after-sales technical support. Do not forcibly start the device in a faulty state.