AI Intelligent Reconnaissance Module

Specification Sheet

Overview

- The AI Intelligent q	Reconnaissance Module is an AI vision-aware tracking kit designed specifically for drones, integrating high-performance target recognition and dynamic tracking. The product integrates a 50MP fixed-focus visible light camera, a 3.3x ultra-clear optical zoom visible light camera, a high-resolution thermal imager, a high-precision laser rangefinder, and is equipped with a lightweight three-axis gimbal and an adaptive shock absorption module, along with auto-focus and gimbal stabilization algorithms to ensure stable and clear high-altitude operation images. At the same time, by integrating deep learning algorithms with high-performance intelligent chips, it can perform tasks such as multi-target recognition and target tracking in visible light images.

Packaging Checklist

- AI main board *1

- Quad camera payload *1

- Power cable *1

- Serial comm.port*1

- Network comm.port*1

AI intelligent recogniction module packing list

Serial comm.cable

Network comm. cable

Power cable

Camera

AI main board

Product Dimensions

AI image processing board

Dimensions (length width height) : 85x56x15mm

4-in-1 gimbal

Infrared camera

Zoom camera

Wide-angle camera

Laser rangefinder

Product Interface

Power input port

Serial Port 2

Network port

Serial Port 1

CAM2

Debug Port

CAM1

Serial 6

Serial 4+5

USB_OTG

USB_HOST

Serial 3

TF Card slot

AI image processing board pin definition

- Network port

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - RX+ | - Default |
| - P2 | - RX- | - Default |
| - P3 | - TX+ | - Default |
| - P4 | - TX- | - Default |

- Serial Port 1

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - VCC | - 5V+ |
| - P2 | - UART3_RX | - 3.3 V |
| - P3 | - UART3_TX | - 3.3 V |
| - P4 | - GND | - GND |

- Serial Port 2

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - VCC | - 5V+ |
| - P2 | - UART2_RX | - 3.3 V |
| - P3 | - UART2_TX | - 3.3 V |
| - P4 | - GND | - GND |

- Power input port

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - VCC | - 12V+ |
| - P2 | - GND | - GND |

- USB_HOST

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - GND | - GND |
| - P2 | - GND | - GND |
| - P3 | - USB2.0 _DP | - Default |
| - P4 | - USB2.0 _DM | - Default |
| - P5 | - VBUS | - 5V+ |

- USB_OTG

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - VBUS | - GND |
| - P2 | - USB2.0 _DM | - Default |
| - P3 | - USB2.0 _DP | - Default |
| - P4 | - GND | - GND |

- Serial Port 3

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - VCC | - 5V+ |
| - P2 | - UART6_RX | - 3.3 V |
| - P3 | - UART6_TX | - 3.3 V |
| - P4 | - GND | - GND |

- Serial port 4+5

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - UART11_RX | - 3.3 V |
| - P2 | - UART11_TX | - 3.3 V |
| - P3 | - UART10_RX | - 3.3 V |
| - P4 | - UART10_TX | - 3.3 V |
| - P5 | - GND | - GND |
| - P6 | - VCC | - 12V+ |

- Serial Port 6

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - UART9_RX | - 12V+ |
| - P2 | - UART9_TX | - 3.3 V |
| - P3 | - GND | - GND |

- CAM1/CAM2

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - D2N | - Default |
| - P2 | - D3N | - Default |
| - P3 | - D2P | - Default |
| - P4 | - D3P | - Default |
| - P5 | - D0N | - Default |
| - P6 | - GND | - GND |
| - P7 | - D0P | - Default |
| - P8 | - GND | - GND |
| - P9 | - GND | - GND |
| - P10 | - D1N | - Default |
| - P11 | - CLKN | - Default |
| - P12 | - D1P | - Default |
| - P13 | - CLKP | - Default |
| - P14 | - MCLK_24M | - 1.8 V |
| - P15 | - GND | - GND |
| - P16 | - GND | - GND |
| - P17 | - PDN | - 1.8 V |
| - P18 | - I2C_SDA | - 1.8 V |
| - P19 | - RST | - 1.8 V |
| - P20 | - I2C_SCL | - 1.8 V |
| - P21 | - VCCIO | - 1.8 V + |
| - P22 | - GND | - GND |
| - P23 | - VCCAF | - 2.8 V + |
| - P24 | - VCC | - 1.2 V + |
| - P25 | - VCC_AF | - 2.8 V + |
| - P26 | - VCC | - 1.2 V + |

- Backup battery port

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - GND | - GND |
| - P2 | - VCC_RTC | - 3.0 V + |

- Fan outlet

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - GND | - GND |
| - P2 | - VCC | - 5.0 V + |

- I/O port

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - VCC | - 5.0 V + |
| - P2 | - I/O1 | - 3.3 V |
| - P3 | - I/O2 | - 3.3 V |
| - P4 | - I/O3 | - 3.3 V |
| - P5 | - I/O4 | - 3.3 V |
| - P6 | - I/O5 | - 3.3 V |
| - P7 | - GND | - GND |

- TF card slot

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - Default | - Default |
| - P2 | - Default | - Default |
| - P3 | - Default | - Default |
| - P4 | - Default | - Default |
| - P5 | - Default | - Default |
| - P6 | - Default | - Default |
| - P7 | - Default | - Default |
| - P8 | - Default | - Default |
| - P9 | - Default | - Default |

Features

- Automatic reconnaissance: Automatically complete reconnaissance of the target area.

- Multi-target recognition: Supports multi-type target recognition and can recognize multiple targets within the frame simultaneously.

- Long-range recognition: Can automatically recognize objects such as human bodies and vehicles within 1500 meters.

- Framing targets: Supports framing targets in visible light and outputting images of the framed objects.

- Locking: Supports manual framing and clicking the recognition box to lock the target.

- Tracking: Supports dynamic target tracking.

- Multi-channel video split-screen display: Supports side-by-side display of infrared split-wide-angle and infrared split-zoom camera videos, and can switch between zoom and wide-angle camera video streams. Zoom control is supported when the zoom camera video stream is displayed.

- Picture-in-picture display: Supports dual-channel picture-in-picture display, with the main picture showing the camera image of primary focus and the secondary picture showing other camera images.

- Precise distance measurement function: Automatically measure the actual distance of the target from the pod when the target is locked; Manual distance measurement is also supported.

- Target location calculation: The longitude and latitude of the target can be calculated based on the GPS location of the drone itself.

- Image storage: The AI image processing board supports the storage of videos recorded during the automatic search process in 1080p format and 30fps frame rate.

Technical Parameters

| AI Image processing board parameters |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Main control computing power (TOPS | 6 |  |  |  |  |  |
| Control Protocol | Private Protocol |  |  |  |  |  |
| Support flight control | PX4 |  |  |  |  |  |
| Input voltage (V) | 12 |  |  |  |  |  |
| Size (length, width, height mm) | 85 * 56 * 15 |  |  |  |  |  |
| Mounting hole positions (mm) | 3.5 * 4 |  |  |  |  |  |
| Overall parameters of the four-light pod |  |  |  |  |  |  |
| Video output interface | Ethernet port |  |  |  |  |  |
| Control Signal Interface | 1 S.BUS, 1 RS422, 1 Ethernet, 1 serial port, 1 CAN |  |  |  |  |  |
| Power supply range | 10V to 28V |  |  |  |  |  |
| System power consumption | Average power consumption: ≤15W Peak power: ≤30W |  |  |  |  |  |
| Protection class | IP4X |  |  |  |  |  |
| Operating temperature | - 20 ℃ ~ 55 ℃ |  |  |  |  |  |
| Size (without shock absorption) | ≤101mm×109mm×150mm |  |  |  |  |  |
| Weight (without shock absorption) | Less than 400g |  |  |  |  |  |
| Mechanical interface type | Quick-release & fixing holes |  |  |  |  |  |
| Zoom camera |  |  |  |  |  |  |
| Image sensor | 48 million pixels 1/2-inch |  |  |  |  |  |
| Maximum photo size | 8000*6000 (4:3) |  |  |  |  |  |
| Video resolution | 3840×2160@30fps 1920×1080@30fps |  |  |  |  |  |
| Encoding format | H.264, H.265 |  |  |  |  |  |
| Focus mode | MF, AFC, AFS |  |  |  |  |  |
| Actual focal length | 15mm-50mm |  |  |  |  |  |
| Equivalent focal length | 81.13 270.44 mm |  |  |  |  |  |
| Aperture | F3.8 - F4.4 |  |  |  |  |  |
| Optical zoom | 3.3 times |  |  |  |  |  |
| Field of view | Focal length | DFOV |  | HFOV |  | VHOV |
|  | Wide | 29 ° |  | 23.5 ° |  | 17.8 ° |
|  | Tele | 9 ° |  | 7.3 ° |  | 5.5 ° |
| Wide-angle camera |  |  |  |  |  |  |
| Image sensor | 50 million pixels, 1/1.3 inches |  |  |  |  |  |
| Maximum photo size | 8192*6144 (4:3) |  |  |  |  |  |
| Video resolution | 3840*2160@30fps 1920*1080@30fps |  |  |  |  |  |
| Encoding format | H.264, H.265 |  |  |  |  |  |
| Focus mode | MF, AFC, AFS |  |  |  |  |  |
| Actual focal length | The 6.55 mm |  |  |  |  |  |
| Equivalent focal length | The 23.1 mm |  |  |  |  |  |
| Aperture | F1.68 |  |  |  |  |  |
| Field of view | DFOV86° HFOV73° VFOV58° |  |  |  |  |  |
| Infrared camera |  |  |  |  |  |  |
| Thermal imaging sensor | Uncooled vanadium oxide infrared focal plane detector |  |  |  |  |  |
| Maximum photo size | 640 * 512 |  |  |  |  |  |
| Video resolution | 640*512@25fps |  |  |  |  |  |
| Encoding format | H.264, H.265 |  |  |  |  |  |
| Pixel spacing | 12 microns |  |  |  |  |  |
| Wavelength range | 8 to 14 microns |  |  |  |  |  |
| Noise equivalent temperature difference | ≤ 50 mk@25℃, f#1.0 |  |  |  |  |  |
| Actual focal length | 18mm |  |  |  |  |  |
| Equivalent focal length | 79mm |  |  |  |  |  |
| Aperture | F1.1 |  |  |  |  |  |
| Field of view | 24.2 ° * 19.5 ° |  |  |  |  |  |
| Laser rangefinder |  |  |  |  |  |  |
| Ranging range | 5-1500m |  |  |  |  |  |
| Ranging accuracy | ≤1m (≤300m) ≤±2m(> 300m) |  |  |  |  |  |
| Working wavelength | 905nm±10nm@25℃ |  |  |  |  |  |
| Ranging frequency | Single, 1Hz, 2Hz, 5Hz |  |  |  |  |  |
| gimbal |  |  |  |  |  |  |
| Stabilization system | 3 Axes (pitch, roll, heading) |  |  |  |  |  |
| Range of rotation | Pitch |  | Roll Over |  | Heading |  |
|  | - 120 ° ~ + 30 ° |  | -90° to +60 |  | + 270 ° |  |
| Stable accuracy | 0.01 ° or less |  |  |  |  |  |
| Working mode | Follow, follow, manual, one-click return to center |  |  |  |  |  |

Notes

Daily maintenance

- Dust and dirt resistant

- Avoid contamination of the lens with chemicals, oil stains, etc. during use. Wipe and clean the lens surface with a lens cloth immediately after use.

- Do not wipe the optical lens directly with your hands or rough cloth. It is recommended to use a professional lens cleaning kit or a lint-free cloth and alcohol for cleaning.

- Moisture-proof and water-resistant

- Avoid prolonged use in rain, snow, high humidity conditions. If in a damp environment, wipe the surface with a dry soft cloth after work and store in a dry environment.

- Do not soak or rinse the equipment directly.

- Anti-collision

- Do not use sharp or hard objects to strike the lens, as this can cause the lens to break and affect its use.

Instructions for Use

- Do not disassemble by yourself

- Non-professionals are not allowed to disassemble core components of the pod (such as optical modules, circuit boards), which may result in damage or disqualification from warranty.

- Anomaly handling

- If the device shows problems such as overheating, noise, image distortion, etc., stop using it immediately and contact after-sales technical support. Do not forcibly start the device in a faulty state.