Optical Fiber Image Transmission  Board Spec Sheet

Overview

This optical fiber board is designed for high performance and high reliability data transmission requirements, featuring an advanced star networking architecture that supports one central node for efficient connection and management of up to 16 sub-nodes, building a stable and scalable network topology.

The video transmission module integrates core communication technologies such as OFDM (Orthogonal Frequency Division Multiplexing) and MIMO (Multiple Input Multiple Output), significantly enhancing spectral efficiency and link reliability. It supports flexible multi-tier bandwidth allocation (1.4MHz, 3MHz, 5MHz, 10MHz, 20MHz) and optimizes resource utilization based on actual application scenarios. At the highest configuration, up to 30Mbps transmission rate can be achieved, effectively reducing system transmission delay and significantly enhancing the overall system's data transmission capacity. Key performance advantages include:

- Long transmission distance: Video transmission supports 20km long transmission, meeting the requirements of long flight.

- High data throughput: With high rate and large bandwidth support, ensure smooth transmission of large amounts of data.

- Outstanding anti-interference performance: Combined with OFDM and MIMO technologies, it ensures stable and reliable communication in complex electromagnetic environments.

- Integrated optical fiber and video transmission: Supports dual-mode transmission of optical fiber and video transmission to meet various application requirements.

Packaging list

- Image transmission optical fiber board - Aerial end *1

- Image transmission optical fiber board - Ground end *1

- Connection cable *2

- Network port communication cables * 2

Image transmission optical fiber board - Groundl end

Image transmission optical fiber board - Aerial end

Network port comm. cables

xConnection cable

Image transmission optical fiber board - Groundl end

Image transmission optical fiber board - Aerial end

Product dimensions and interfaces

Mechanical parameters

Product size (length X width) : 90.95mm x 84.47mm

Power port

Interface definitions and functions

Fiber optic port

Network Port 1

Port 2

Power port pin definition

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - UART_TX | - 3.3 V |
| - P2 | - UART_RX | - 3.3 V |
| - P3 | - GND | - GND |
| - P4 | - VCC | - 12V+ |

Port 1 pin definition

| - Pin | - Signal | - Voltage |
| --- | --- | --- |
| - P1 | - TX- | - Default |
| - P2 | - TX+ | - Default |
| - P3 | - RX- | - Default |
| - P4 | - RX+ | - Default |

Functional features

- Star networking with key technologies such as OFDM and MIMO

- Dual-antenna: single-antenna transmission, dual-antenna reception

- Supported wireless operating bands: 2401.5-2481.5MHz, 1427.9-1447.9MHz, 806-826 MHZ

- Wireless bandwidth can be configured: 1.4 MHz / 3 MHz / 5 MHz / 10 MHz / 20 MHz, two nodes support one-way maximum 30 MBPS

- Modulation: Supports QPSK, 16QAM, 64QAM modulation

- Encryption options: ZUC, SNOW3G, AES

- Maximum transmission power: 25 DBM ±2

- Maximum transmission distance: 20km

- Node capacity: Up to 16 access nodes

- Configuration management: Fulfill basic configuration/reporting/query functionality via WEB UI

- Fiber optic module: Supports stable fiber transmission up to 20km

Technical Parameters

| - Module Category | - Parameter Categories | - Parameter items | - Detailed parameters |
| --- | --- | --- | --- |
| - Basic Parameters |  | - Supply voltage | - 12V |
|  |  | - Weight | - 48g |
|  |  | - Size (length X width mm) | - 90.95 X84.47 |
| - Fiber optic transceiver | - Interface |  | - FC interface |
|  | - Transmission distance |  | - 0-20km |
|  | - Data transfer rate |  | - 155Mbps |
| - Wireless video transmission | - RF | - Power rating | 2.4G/1.4G/800M, 25dBm±2 |
|  |  | - Radio frequency band | - 2401.5 to 2481.5 MHz - 1427.9 to 1447.9 MHz - 806-826MHz |
|  |  | - Sensitivity (access) | - 2.4G (24415 channel number) : - 20MHz -99dBm - 10MHz -103dBm - 5MHz -104dBm - 3MHz-106dBm |
|  |  |  | - 1.4G (14379 channel number) - 20MHz -99dBm - 10MHz -103dBm - 5MHz -104dBm - 3MHz -106dBm |
|  |  |  | - 800M (8160 channel number) - 20MHz -99dBm - 10MHz -103dBm - 5MHz -104dBm - 3MHz -106dBm |
|  |  | - Sensitivity (BLER<=3%) | - 2.4G (24415 frequency points) : - 20MHz -97dBm(5Mbps) - 10MHz -96dBm(5Mbps) - 5MHz -93dBm(5Mbps) - 3MHz -98dBm(2Mbps) |
|  |  |  | - 1.4G (14379 frequency points) : - 20MHz -97dBm(5Mbps) - 10MHz -96dBm(5Mbps) - 5MHz -91dBm(5Mbps) - 3MHz -97dBm(2Mbps) |
|  |  |  | - 800M (8160 points) : - 20MHz -97dBm(5Mbps) - 10MHz -97dBm(5Mbps) - 5MHz -94dBm(5Mbps) - 3MHz -98dBm(2Mbps |
|  | - Transmission mode | - Unicast, multicast, broadcast | - Support |
|  | - Transmission mode | - Transmission mode | - Single-antenna transmission, dual-antenna reception |
|  |  | - Data link | - Master-slave two-way data communication |
|  |  | - Uplink and downlink matching | - The uplink and downlink time slot ratio of the central node supports 2D3U/3D2U/4D1U/1D4U |
|  | - Operating bandwidth | - Support bandwidth | - 1.4 MHz / 3 MHz / 5 MHz / 10 MHz / 20 MHz |
|  | - Rate | - Peak rate | - Two nodes support a one-way maximum of 30Mbps |
|  |  | - Rate levels | - Adaptive average distribution system rate |
|  | - Encryption | - Encryption algorithm | - ZUC, SNOW3G, AES - three encryption options available |
|  | - Modulation | - Modulation method | - Supports QPSK, 16QAM, 64QAM modulation |
|  | - Anti-interference | - Dynamic frequency modulation | - Support frequency hopping |
|  | - Retransmission | - HARQ retransmit | - Support |
|  | - Network capacity | - Number of network nodes | - Up to 16 access nodes |
|  | - Sleep | - DRX | - Master node control, slave nodes allow access to DRX, 160ms cycle wake-up |
|  | - Transmission distance | - Limit transmission distance | - 20km |
|  | - Delay | - Module air interface delay | - UL is unidirectional, with a delay of no more than 15ms - DL unidirectional, delay <=15ms |
|  |  | - Startup delay | - Central node/slave node startup delay 15 seconds |
|  | - System control | - Parameter configuration | - Transmit power/frequency point/bandwidth (change in real time), band (change not in real time) |
|  |  | - Status/Parameter reporting | - Connection status /rsrp/ snr/distance/ uplink/downlink throughput, etc |
|  | - Configuration Management | - WEBUI | - Satisfy basic configuration/reporting/query functionality |
|  | - Software upgrade | - OTA | - Support OTA upgrades, support local and remote upgrades |
|  | - Transmitter spurious | - <-36dBm@BW=1kHz | - 9KHz=<f<150KHz |
|  |  | - <-36dBm@BW=10kHz | 150KHz=<f<30MHz |
|  |  | <-36dBm@BW=100kHz | 30MHz=<f<1000MHz |
|  |  | <-36dBm@BW=1MHz | 1 GHz = < f < 12.75 GHz |
|  | - Adjacent channels - Leak ratio | - ACLR | - < 29.2 dBc @ E - UTRA1 - < 32.2 dBc @ UTRA1 - < 35.2 dBc @ UTRA2 |
|  | - Power consumption | - Peak | - Power consumption value at maximum rate: 700mA±15% |
|  | - Temperature range | - Operating temperature | - -20°C to 75°C |
|  |  | - Storage temperature | - -40°C to 85°C |
|  | - Humidity range | - Working humidity | - 5% to 95% (no condensation) |